# TaoTao Controller Communication Protocol  V1.1.29

TaoTao Intelligent Technology Co., Ltd.

Electric Bicycles and Scooters

Communication Protocol Design Specification

| Version | Description | Revised By | Date |
| --- | --- | --- | --- |
| V1.0 | Created | TaoTao Intelligent | 2020-01-03 |
| V1.1 | Reorganized | Zou Fuchen | 2020-10-12 |
| V1.1.1 | Added seven-color light control bit in heartbeat packet | Zou Fuchen | 2020-10-27 |
| V1.1.24 | Added new function code for color lights; 0x0020 changed to 0x001A | Zou Fuchen | 2020-11-26 |
| V1.1.25 | Battery communication 0x0020–0x002F, refer to <TaoTao Battery Communication Protocol> | Zou Fuchen | 2020-12-07 |
| V1.1.26 | In heartbeat, Data 5 Bit7 reserved changed to motion mode switch |  | 2021-07-28 |
| V1.1.27 | In 0x01 heartbeat reply, Data 4 Bit5 reserved changed to gyroscope angle protection status; 0x03 parameter configuration data Byte1Bit3 reserved bit changed to gyroscope calibration bit |  | 2022-11-25 |
| V1.1.28 | Simplified |  |  |

# Table of Contents

1.  Communication Protocol

1.1.  Overview

1.2.  Protocol Format

1.2.1.  Frame Header

1.2.2.  Device Address. Read/Write Permissions

1.2.3.  Function Code

1.2.4.  Data Length

1.2.5.  Data

1.2.6.  CRC16 Checksum

2.  Function Codes

2.1.  Detailed Function Description

2.1.1.  Controller Type Query  0x0000

2.2.2.  Heartbeat Data Exchange (recommended every 20 ms)  0x0001

2.2.3.  Controller Status Query/Set  0x0002

2.2.7.  Parameter Configuration Query/Set  0x0003

2.2.8.  Light Control Query/Set  0x0004

2.2.9.  Gear Speed Query/Set  0x0005

2.2.10.  Auto Power-Off Time Configuration Query/Set  0x0006

2.2.11.  Real-Time Clock Query/Set Controller  0x0007

2.2.12.  Single-Trip Distance Query Controller  0x0008

2.2.13.  Total Distance Query Controller  0x0009

2.2.14.  Temperature Query Controller  0x000A

2.2.15.  Driving Current Query/Set Controller  0x000B

2.2.16.  Battery Voltage Query Controller  0x000C

2.2.17.  Battery Total Capacity Query Controller  0x000D

2.2.18.  Battery Remaining Capacity Query/Set Controller  0x000E

2.2.19.  Battery Charge/Discharge Count Query/Set Controller  0x000F

2.2.20.  Battery Manufacturer Code and Number Query/Set Controller  0x0010

2.2.21.  Controller Manufacturer Code and HW/SW Version Query/Set  0x0011

2.2.22.  Geared Motor Hall Pulse Count Query/Set Controller  0x0012

2.2.23.  Geared Motor Hall Pulse Interval Time (unit ms) Query  0x0013

2.2.24.  Motor Parameters Query/Configure  0x0014

2.2.25.  Wheel Diameter Query/Set Controller  0x0015

2.2.26.  FLASH Data Version Query  0x0016

2.2.27.  Manufacturer Code and HW/SW Version Query  0x0017

2.2.28.  Instrument Backlight Brightness Query/Set  0x0018

2.2.29.  Night Mode Query/Set  0x0019

2.2.30.  Color Light Settings  0x001A

2.2.31.  SN Serial Number Query  0x001D

2.2.32.  Battery Status Query  0x0020

2.2.33.  Gyroscope Angle Query  0x0021

2.2.34.  BMS Firmware Version Query  0x002A

2.2.35.  Single-Trip Distance Query  0x0030

2.2.36.  Single-Trip Time Query  0x0031

2.2.37.  Speed Query  0x0032

2.2.38.  Incoming Call Push  0x00A0

2.2.39.  SMS Push  0x00A1

2.2.40.  Message Push  0x00A2

2.2.41.  Time Push  0x00A3

2.2.42.  Bluetooth Password Query/Configure  0x00A4

2.2.43.  Navigation Push  0x00A5

2.2.44.  Reset Command (Restore Factory Settings)  0x00A6

2.2.45.  Lock/Unlock Setting  0x00A7

2.2.46.  Lock Password Modification  0x00A8

2.2.47.  After-Sales Password Query  0x00A9

2.2.48.  Weather Information Push  0x00AA

2.2.49.  Query Slave (Bootloader) Upgrade Conditions  0x00D0

2.2.50.  Transmit Upgrade Data Package / Query Receive Status  0x00D1

2.2.51.  Upgrade Verification / Upgrade End Response  0x00D2

2.2.52.  Query Slave (Application) Upgrade Conditions  0x00E0

2.2.53.  Transmit Upgrade Data Package / Query Receive Status  0x00E1

2.2.54.  Upgrade Verification / Upgrade End Response  0x00E2

2.2.55.  Query Slave (Flash) Upgrade Conditions  0x00F0

2.2.56.  Transmit Upgrade Data Package / Query Receive Status  0x00F1

2.2.57.  Upgrade Verification / Upgrade End Response  0x00F2

3.  Examples

3.1.  Heartbeat Data Exchange

3.1.1.  IOT or Bluetooth Instrument Sends

3.1.2.  Controller Reply

3.2.  Query/Configure Controller Status

3.2.1.  IOT or Bluetooth Instrument Sends

3.2.2.  Controller Reply

3.3.  Query/Configure Controller Light Control

3.4.  OTA Communication Commands

4.  Notes

4.1.  CRC16 Reference Algorithm

4.2.  CRC32 Reference Algorithm

# 1.  Communication Protocol

## 1.1.  Overview

This communication protocol is used for communication between the controller, instrument, IOT module, or Bluetooth module.

The protocol is designed with reliability, security, efficiency, and extensibility in mind.

Communication interface: UART  Baud rate: 38400, 1 start bit, 8 data bits, 1 stop bit, no parity. Multi-byte data is transmitted high byte first, low byte last

Communication uses a request-response mode: the instrument, IOT module, or Bluetooth module sends a request; the controller replies.

If the IOT or Bluetooth module is in controller-upgrade mode and no response is received within 250 ms, the current data is re-sent. After 20 re-sends without a response, the upgrade is abandoned, a fault is indicated, and the user is prompted to retry the upgrade.

If an instrument acts as a bridge, the IOT or Bluetooth module uses the instrument as the control-response end.

Added read/write permissions to distinguish read/write operations and prevent modification of sensitive data.

## 1.2.  Protocol Format

| Data[0] Frame Header | Data[1] Device Address | Data[2] Function Code | Data[3] Reserved | Data[4] Data Length | Data[5]–Data[n] Data | Data[n+1] Data[n+2] CRC16 Checksum |
| --- | --- | --- | --- | --- | --- | --- |
| 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | n Byte n<=130 | 2 Byte |

Table 1

### 1.2.1.  Frame Header

Frame header Data[0] (1 Byte): fixed value 0x5A, indicates the start of a frame.

### 1.2.2.  Device Address. Read/Write Permissions

Device address Data[1] (1 Byte): combined definition that includes master/slave identification, read/write permission (configured per actual requirements to prevent modification of sensitive parameters), and slave device address.

| Field Name |  | Description |  | Notes |
| --- | --- | --- | --- | --- |
| Device Address | Bit7 | Master/Slave ID | 0: Master, 1: Slave |  |
|  | Bit4–6 | Read/Write Permission | 000: Read-only 001: Write, no response required 010: Write, response required 011: Response |  |
|  | Bit0–3 | Slave Device Address | 0001: Controller 0010: Battery 0011: Instrument |  |

Table 2

### 1.2.3.  Function Code

Function code (Data[2]) (2 Bytes): defines the meaning of the data field.

Data[3] is reserved; it carries the high byte of the function code.

### 1.2.4.  Data Length

Data length (1 Byte): the length of data in the command (does not include frame header, device address, function code, data length, or checksum).

Data Packet Special Notes:

1. Heartbeat data packet – recommended to be sent every 20 ms; contains real-time sensitive information and serves as the primary communication data.

2. Single-operation data packet – sent as required by the device (see the appendix for the specific function list). If a Byte consists of bit-combined data, it includes an effective-data-declaration mask Byte (shown in the appendix table). This mask Byte declares which data bits need to be read or modified: mask Bit = 1 means the corresponding data bit is valid; mask Bit = 0 means the data is masked/ignored. If a Byte is an independent parameter, no effective-data declaration is needed; all data bits are valid.

3. Data for multiple function codes can be transferred and modified in one packet; the specific data length is set according to actual size.

Table 3

### 1.2.5.  Data

Data (N Bytes): the data that needs to be transmitted in the command.

### 1.2.6.  CRC16 Checksum

CRC16 checksum (2 Bytes): CRC16 checksum of all preceding bytes, including frame header, device address, function code, data length, and data. When either side receives a data packet, it performs CRC verification; if incorrect, the packet is discarded.

Supplementary note: In specific scenarios, a slave device is allowed to proactively report data, i.e., even if the master has not queried a parameter, the slave may actively push the relevant parameter to the master.

# 2.  Function Codes

## 2.1.  Detailed Function Description

### 1.  Controller Type Query  0x0000

Master Side (IOT / BLE / Meter)

| Data[0] Frame Header | Data[1] Device Address ID | Data[2] Function Code | Data[3] Reserved | Data[4] Data Length | Data[5]~Data[n] Data | Data[n+1] Data[n+2] CRC16 |
| --- | --- | --- | --- | --- | --- | --- |
| 0x5A | 0xXX | 0x00 | 0x00 | 0x00 | None | 0xXX, 0xXX |

Device Address: 0x01  Binary 0 000 0001 – master reads controller type only

Function Code: 0x00 00

Data Length: 0

Data: None

0x5A + ID + 0x00 0x00 0x00 0xXX 0xXX

Slave Side (Controller / Instrument / Battery)

Device Address: 0xB1  Binary 1 011 0001 – slave response

Function Code: 0x00 00

Data Length: 1

Data: 0x00 = Master directly connected to controller, scooter controller

0x5A + ID + 0x00 0x00 0x01 + Data + CRC

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit7 | Receiver type: 0 = Direct connection to controller, 1 = Instrument bridging |
|  | Bit6 | Reserved |
|  | Bit5 | Reserved |
|  | Bit4 | Reserved |
|  | Bit3–Bit2–Bit1–Bit0 | Controller type: 0 = Scooter controller, 1 = PAS bicycle controller, 2 = Balance scooter controller |

### 2.  Heartbeat Data Exchange  (recommended every 20 ms)  0x0001

Master Side (IOT / BLE / Meter)

Device Address: 0xXX  →  refer to Device Address / Read-Write Permissions

Function Code: 0x00 01

Data Length: 0x08

Data:

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit4–7 | Reserved |
|  | Bit0–3 | Throttle AD sample value – high 4 bits |
| Byte6 | Bit0–7 | Throttle AD sample value – low 8 bits |
| Byte7 | Bit5–7 | Reserved |
|  | Bit4 | Mechanical brake status: 0 = Not braking, 1 = Braking |
|  | Bit0–3 | Electronic brake AD sample value – high 4 bits |
| Byte8 | Bit0–7 | Electronic brake AD sample value – low 8 bits |
| Byte9 | Bit7 | Motion mode: 0 = Normal, 1 = Sport mode (updated 2021-07-28, supports single/dual drive) |
|  | Bit6 | Cruise control: 0 = Off, 1 = On |
|  | Bit5 | Unit switch – metric/imperial: 0 = Metric (km), 1 = Imperial (miles) |
|  | Bit4 | Speed display switch: 0 = Show km/h (or miles), 1 = Show single-revolution time (ms) |
|  | Bit3 | Under-voltage status: 0 = Normal, 1 = Under-voltage |
|  | Bit0–2 | Gear level (0–7) |
| Byte10 | Bit7 | Horn status: 0 = Silent, 1 = Sounding |
|  | Bit6 | Start mode: 0 = Zero-start, 1 = Non-zero-start |
|  | Bit5 | Lock status: 0 = Unlocked, 1 = Locked |
|  | Bit4 | Walk-assist mode: 0 = Normal riding, 1 = Walk-assist |
|  | Bit3 | Headlight switch: 0 = Off, 1 = On |
|  | Bit0–2 | Power state: 0x00 = Off, 0x01 = On, 0x02 = Power-off command |
| Byte11 | Bit3–7 | (For instrument) Ambient light color switch |
|  | Bit1–2 | (For instrument) Ambient light mode: 0 = Constant on, 1 = Seven-color running lights (two modes currently) |
|  | Bit0 | (For instrument) Ambient light switch: 0 = Off, 1 = On |
| Byte12 | Bit0–7 | Reserved |

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX  →  refer to Device Address / Read-Write Permissions

Function Code: 0x00 01

Data Length: 0x0A

Data:

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | Vehicle battery percentage (0–100) |
| Byte6 | Bit7 | Speed display switch: 0 = Report metric speed (unit 0.1 km/h), 1 = Report wheel revolution time (ms) |
|  | Bit0–6 | Vehicle real-time speed high 7 bits (0.1 km/h) / revolution time high 7 bits (ms) |
| Byte7 | Bit0–7 | Vehicle real-time speed low 8 bits (0.1 km/h) / revolution time low 8 bits (ms) |
| Byte8 | Bit7 | Daytime running light status: 0 = Off, 1 = On |
|  | Bit6 | Gyroscope hill-climbing status: 0 = Not climbing, 1 = Climbing |
|  | Bit5 | Gyroscope protection status: 0 = Not protected, 1 = Protected |
|  | Bit4 | Cruise control: 0 = Off, 1 = On |
|  | Bit3 | Unit switch: 0 = Metric, 1 = Imperial |
|  | Bit0–2 | Gear level (0–7) |
| Byte9 | Bit7 | Under-voltage status: 0 = Normal, 1 = Under-voltage |
|  | Bit6 | Assist status: 0 = No assist, 1 = Assisting |
|  | Bit5 | Cruise status: 0 = Not cruising, 1 = Cruising |
|  | Bit4 | Charging status: 0 = Not charging, 1 = Charging |
|  | Bit3 | Throttle valid: 0 = Invalid, 1 = Valid (voltage > 1.2 V) |
|  | Bit2 | Electronic brake status: 0 = Not braking, 1 = Braking |
|  | Bit1 | Mechanical brake status: 0 = Not braking, 1 = Braking |
|  | Bit0 | Motor running status: 0 = Motor not running, 1 = Motor running |
| Byte10 | Bit7 | Horn status: 0 = Silent, 1 = Sounding |
|  | Bit6 | Start mode: 0 = Zero-start, 1 = Non-zero-start |
|  | Bit5 | Lock status: 0 = Unlocked, 1 = Locked |
|  | Bit4 | Walk-assist mode: 0 = Normal riding, 1 = Walk-assist |
|  | Bit3 | Headlight switch: 0 = Off, 1 = On |
|  | Bit0–2 | Power state: 0x00 = Off, 0x01 = On, 0x02 = Power-off command |
| Byte11 | Bit7 | Gyroscope fault: 0 = No fault, 1 = Fault |
|  | Bit6 | Battery fault: 0 = No fault, 1 = Fault |
|  | Bit5 | Controller fault: 0 = No fault, 1 = Fault |
|  | Bit4 | Motor phase wire or MOS short-circuit fault: 0 = No fault, 1 = Fault |
|  | Bit3 | Motor Hall sensor fault: 0 = No fault, 1 = Fault |
|  | Bit2 | Brake fault: 0 = No fault, 1 = Fault |
|  | Bit1 | Throttle fault: 0 = No fault, 1 = Fault |
|  | Bit0 | Communication fault: 0 = No fault, 1 = Fault |
| Byte12 | Bit0–7 | Battery real-time voltage high byte (unit 0.1 V) |
| Byte13 | Bit0–7 | Battery real-time voltage low byte (unit 0.1 V) |
| Byte14 | Bit0 | Battery under-voltage: 0 = No fault, 1 = Fault |
|  | Bit1 | Battery over-temperature: 0 = No fault, 1 = Fault |
|  | Bit2 | Controller temperature protection: 0 = No fault, 1 = Fault |
|  | Bit3–6 | Reserved |
|  | Bit7 | Tire pressure module fault: 0 = No fault, 1 = Fault |

### 3.  Controller Status Query/Set  0x0002

Master Side (IOT / BLE / Meter)

Device Address: 0xXX  →  refer to Device Address / Read-Write Permissions

Function Code: 0x00 02

Data Length: 0x02

Data:

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit7 | Unit switch – metric/imperial: 0 = Metric, 1 = Imperial |
|  | Bit6 | Bluetooth status: 0 = Not connected, 1 = Connected |
|  | Bit5 | Brake mode: 0 = Power-cut braking, 1 = Electronic braking |
|  | Bit4 | Auto power-off: 0 = Disabled, 1 = Enabled |
|  | Bit3 | Assist mode: 0 = Non-assist, 1 = Assist |
|  | Bit2 | Cruise control: 0 = Off, 1 = On |
|  | Bit1 | Start mode: 0 = Zero-start, 1 = Non-zero-start |
|  | Bit0 | Lock status: 0 = Unlocked, 1 = Locked |
| Byte6 | Bit0–7 | Mask bits (0 = masked, 1 = valid / unmasked) corresponding to Byte1 bits 0–7 |

Slave Side (Controller / Instrument / Battery)

Device Address: 0xB1  →  refer to Device Address / Read-Write Permissions

Function Code: 0x00 02

Data Length: 0x02

Data:

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit7 | Unit switch – metric/imperial: 0 = Metric, 1 = Imperial |
|  | Bit6 | Bluetooth status: 0 = Not connected, 1 = Connected |
|  | Bit5 | Brake mode: 0 = Power-cut braking, 1 = Electronic braking |
|  | Bit4 | Auto power-off: 0 = Disabled, 1 = Enabled |
|  | Bit3 | Assist mode: 0 = Non-assist, 1 = Assist |
|  | Bit2 | Cruise control: 0 = Off, 1 = On |
|  | Bit1 | Start mode: 0 = Zero-start, 1 = Non-zero-start |
|  | Bit0 | Lock status: 0 = Unlocked, 1 = Locked |
| Byte6 | Bit0–7 | Mask bits (0 = masked, 1 = valid / unmasked) corresponding to Byte1 bits 0–7 |

### 4.  Parameter Configuration Query/Set  0x0003

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 03

Data Length: 0x02

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit7 | Reserved |
|  | Bit6 | Reserved |
|  | Bit5 | Reserved |
|  | Bit4 | NFC function On/Off: 0 = Off, 1 = On |
|  | Bit3 | Gyroscope calibration: 0 = No action, 1 = Ready to calibrate (keep vehicle stationary) |
|  | Bit2 | Clear riding time: 0 = No action, 1 = Clear |
|  | Bit1 | Clear total mileage: 0 = No action, 1 = Clear |
|  | Bit0 | Restore factory settings: 0 = No action, 1 = Restore |
| Byte6 | Bit0–7 | Mask bits corresponding to Byte1 bits 0–7 |

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 03

Data Length: 0x02

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit7 | Reserved |
|  | Bit6 | Reserved |
|  | Bit5 | Reserved |
|  | Bit4 | Reserved |
|  | Bit3 | Gyroscope calibration: 0 = No action or failed, 1 = Calibration successful |
|  | Bit2 | Clear riding time: 0 = No action, 1 = Cleared |
|  | Bit1 | Clear total mileage: 0 = No action, 1 = Cleared |
|  | Bit0 | Restore factory settings: 0 = No action, 1 = Restored |
| Byte6 | Bit0–7 | Mask bits corresponding to Byte1 bits 0–7 |

### 5.  Light Control Query/Set  0x0004

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 04

Data Length: 0x02

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit7 | Reserved |
|  | Bit6 | Reserved |
|  | Bit5 | Front light switch: 0 = Off, 1 = On |
|  | Bit4 | Brake light switch: 0 = Off, 1 = On |
|  | Bit3 | Ambient light switch: 0 = Off, 1 = On |
|  | Bit2 | Right turn signal: 0 = Off, 1 = On |
|  | Bit1 | Left turn signal: 0 = Off, 1 = On |
|  | Bit0 | Daytime running light switch: 0 = Off, 1 = On |
| Byte6 | Bit0–7 | Mask bits corresponding to Byte1 bits 0–7 |

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 04

Data Length: 0x02

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit7 | Reserved |
|  | Bit6 | Reserved |
|  | Bit5 | Front light switch: 0 = Off, 1 = On |
|  | Bit4 | Brake light switch: 0 = Off, 1 = On |
|  | Bit3 | Ambient light switch: 0 = Off, 1 = On |
|  | Bit2 | Right turn signal: 0 = Off, 1 = On |
|  | Bit1 | Left turn signal: 0 = Off, 1 = On |
|  | Bit0 | Daytime running light switch: 0 = Off, 1 = On |
| Byte6 | Bit0–7 | Mask bits corresponding to Byte1 bits 0–7 |

### 6.  Gear Speed Query/Set  0x0005

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 05

Data Length: 0x02

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit7 | Hidden mode gear: 1 = On, 0 = Off |
|  | Bit3–6 | Gear type, 0–15 0: Binary x0000xxx – Vehicle speed upper limit (read-only speed) 1: Binary x0001xxx 2: Binary x0010xxx 3: Binary x0011xxx – Maximum speed limit; set or read target gear max speed 4: Binary x0100xxx – Switch current gear (Bit0–2, 8 gear levels) 5: Binary x0101xxx – Energy recovery (Bit0–2, 8 intensity levels) 6: Binary x0110xxx – Drive mode (Bit0–2: 1 = Front drive, 2 = Front+rear drive, 3 = Rear drive) |
|  | Bit0–2 | Target gear to set (0–7) |
| Byte6 | Bit0–7 | Speed value for the gear (unit: km/h) |

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 05

Data Length: 0x02

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit7 | Hidden mode gear: 1 = On, 0 = Off |
|  | Bit3–6 | Gear type: 0: Throttle mode gear 1: Assist mode gear 2: Walk-assist mode gear 3: Maximum speed limit 4: Current gear 5: Energy recovery 6: Drive mode (Bit0–2: 1 = Front drive, 2 = Front+rear drive, 3 = Rear drive) |
|  | Bit0–2 | Gear to set (0–7) |
| Byte6 | Bit0–7 | Speed for the gear (unit: km/h) |

### 7.  Auto Power-Off Time Configuration Query/Set  0x0006

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 06

Data Length: 0x02

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | Auto power-off time high byte (unit: s) |
| Byte6 | Bit0–7 | Auto power-off time low byte (unit: s) |

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 06

Data Length: 0x02

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | Auto power-off time high byte (unit: s) |
| Byte6 | Bit0–7 | Auto power-off time low byte (unit: s) |

### 8.  Real-Time Clock Query/Set Controller  0x0007

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 07

Data Length: 0x02

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | Hour (H) |
| Byte6 | Bit0–7 | Minute (min) |

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 07

Data Length: 0x02

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | Hour (H) |
| Byte6 | Bit0–7 | Minute (min) |

### 9.  Single-Trip Distance Query Controller  0x0008

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 08

Data Length: 0x00

Data: None

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 08

Data Length: 0x02

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | Single-trip distance high byte (16-bit unsigned, high byte first, unit 0.1 km) |
| Byte6 | Bit0–7 | Single-trip distance low byte (16-bit unsigned, high byte first, unit 0.1 km) |

### 10.  Total Distance Query Controller  0x0009

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 09

Data Length: 0x00

Data: None

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 09

Data Length: 0x04

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5–8 | Bit0–31 | Total distance (32-bit unsigned, high byte first, unit 0.1 km) |

### 11.  Temperature Query Controller  0x000A

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 0A

Data Length: 0x00

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit4–7 | Device type: 0 = Controller, 1 = Internal battery, 2 = External battery, 3 = Motor temperature |
|  | Bit0–3 | Temperature number: 0 = Temp 1, 1 = Temp 2, … |

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 0A

Data Length: 0x03

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit4–7 | Device type: 0 = Controller, 1 = Internal battery, 2 = External battery, 3 = Motor temperature |
|  | Bit0–3 | Temperature number: 0 = Temp 1, 1 = Temp 2, … |
| Byte6 | Bit0–7 | Temperature (unit °C, 8-bit unsigned; actual temperature = value − 60°C) |

### 12.  Driving Current Query/Set Controller  0x000B

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 0B

Data Length: 0x03

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit2–7 | Reserved |
|  | Bit0–1 | Current type: 0 = Query real-time driving current, 1 = Query controller current limit, 2 = Set current |
| Byte6 | Bit0–7 | Current high byte (16-bit signed, unit 0.1 A; positive = discharging, negative = charging) |
| Byte7 | Bit0–7 | Current low byte (16-bit signed, unit 0.1 A; positive = discharging, negative = charging) |

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 0B

Data Length: 0x03

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit1–7 | Reserved |
|  | Bit0 | Current type: 0 = Real-time driving current, 1 = Controller current limit |
| Byte6 | Bit0–7 | Current high byte (16-bit signed, unit 0.1 A) |
| Byte7 | Bit0–7 | Current low byte (16-bit signed, unit 0.1 A) |

### 13.  Battery Voltage Query Controller  0x000C

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 0C

Data Length: 0x01

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit1–7 | Reserved |
|  | Bit0 | Battery type: 0 = Internal battery, 1 = External battery |

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 0C

Data Length: 0x03

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit1–7 | Reserved |
|  | Bit0 | Battery type: 0 = Internal, 1 = External |
| Byte6 | Bit0–7 | Battery voltage high byte (unit 0.1 V) |
| Byte7 | Bit0–7 | Battery voltage low byte (unit 0.1 V) |

### 14.  Battery Total Capacity Query Controller  0x000D

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 0D

Data Length: 0x01

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit1–7 | Reserved |
|  | Bit0 | Battery type: 0 = Internal, 1 = External |

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 0D

Data Length: 0x04

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit1–7 | Reserved |
|  | Bit0 | Battery type: 0 = Internal, 1 = External |
| Byte6 | Bit0–7 | Battery total capacity high byte (unit mAh) |
| Byte7 | Bit0–7 | Battery total capacity low byte (unit mAh) |
| Byte8? | Bit0–7 | Battery health factor 0–100, default 100 |

### 15.  Battery Remaining Capacity Query/Set Controller  0x000E

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 0E

Data Length: 0x02

| Byte | Bit | Description |
| --- | --- | --- |
| Byte1 | Bit1–7 | Reserved |
|  | Bit0 | Battery type: 0 = Internal, 1 = External |

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 0E

Data Length: 0x03

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit1–7 | Reserved |
|  | Bit0 | Battery type: 0 = Internal, 1 = External |
| Byte6 | Bit0–7 | Battery remaining capacity high byte (unit mAh) |
| Byte7 | Bit0–7 | Battery remaining capacity low byte (unit mAh) |

### 16.  Battery Charge/Discharge Count Query/Set Controller  0x000F

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 0F

Data Length: 0x01

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit1–7 | Reserved |
|  | Bit0 | Battery type: 0 = Internal, 1 = External |

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 0F

Data Length: 0x03

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit1–7 | Reserved |
|  | Bit0 | Battery type: 0 = Internal, 1 = External |
| Byte6 | Bit0–7 | Battery charge/discharge count high byte |
| Byte7 | Bit0–7 | Battery charge/discharge count low byte |

### 17.  Battery Manufacturer Code and Number Query/Set Controller  0x0010

Master Side (IOT / BLE / Meter)

| Data[0] Frame Header | Data[1] Device Address | Data[2] Function Code | Data[3] Reserved | Data[4] Data Length | Data[5]~Data[n] Data | Data[n+1] Data[n+2] CRC16 |
| --- | --- | --- | --- | --- | --- | --- |
| 0x5A | 0xXX | 0x10 | 0x00 | 0x01 |  | 0xXX, 0xXX |

Device Address: 0xXX

Function Code: 0x00 10

Data Length: 0x01

| Byte | Bit | Description |
| --- | --- | --- |
| Byte1 | Bit1–7 | Reserved |
|  | Bit0 | Battery type: 0 = Internal, 1 = External |

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 10

Data Length: 0xXX (≤ 100 bytes)

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit1–7 | Reserved |
|  | Bit0 | Battery type: 0 = Internal, 1 = External |
| Byte6–n | Bit0–7 | Battery manufacturer code and number |

### 18.  Controller Manufacturer Code and HW/SW Version Query/Set  0x0011

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 11

Data Length: 0x00

Data: None

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 11

Data Length: 0x0C

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5–8 | Bit0–7 | Controller manufacturer code (ASCII digits or letters) |
| Byte9 | Bit0–7 | Hardware version integer byte: 0x99 = 99 |
| Byte10 | Bit0–7 | Hardware version decimal byte (1 = 0.01): 0x99 = 0.99 |
| Byte11 | Bit0–7 | Software version integer byte: 0x99 = 99 |
| Byte12 | Bit0–7 | Software version decimal byte (1 = 0.01): 0x99 = 0.99 |
| Byte13–16 | Bit0–7 | Controller manufacturer code (ASCII digits or letters) |

### 19.  Geared Motor Hall Pulse Count Query/Set Controller  0x0012

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 12

Data Length: 0x00

Data: None

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 12

Data Length: 0x04

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit4–7 | Reserved |
|  | Bit0–3 | Unit time: 0 = Gearless motor (ignore), 1 = 100 ms, 2 = 250 ms, 3 = 500 ms, 4 = 750 ms, 5 = 1 s, 6 = 5 s, 7 = 1 min… |
| Byte6–7 | Bit0–23 | Number of geared motor pulse waves received within the unit time |

### 20.  Geared Motor Hall Pulse Interval Time Query  0x0013

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 13

Data Length: 0x00

Data: None

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 13

Data Length: 0x02

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | Geared motor Hall pulse interval time high 8 bits (unit: ms) |
| Byte6 | Bit0–7 | Geared motor Hall pulse interval time low 8 bits (unit: ms) |

### 21.  Motor Parameters Query/Configure  0x0014

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 14

Data Length: 0x03

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | Motor pole pair count |
| Byte6–7 | Bit0–15 | Motor gear ratio (unit: 0.1) |

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 14

Data Length: 0x03

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | Motor pole pair count |
| Byte6–7 | Bit0–15 | Motor gear ratio (unit: 0.1) |

### 22.  Wheel Diameter Query/Set Controller  0x0015

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 15

Data Length: 0x02

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit7 | Wheel diameter unit switch: 0 = Inches, 1 = Centimeters |
|  | Bit0–6 | Size in inches (unit 0.1 inch) / size in centimeters (unit 0.1 cm) |
| Byte6 | Bit0–7 | (additional size data) |

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 15

Data Length: 0x02

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit7 | Wheel diameter unit switch: 0 = Inches, 1 = Centimeters |
|  | Bit0–6 | Size in inches (unit 0.1 inch) / size in centimeters (unit 0.1 cm) |
| Byte6 | Bit0–7 | (additional size data) |

### 23.  FLASH Data Version Query  0x0016

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 16

Data Length: 0x00

Data: None

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 17

Data Length: 0x08

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5–8 | Bit0–31 | Manufacturer code (ASCII digits or letters) |
| Byte9–11 | Bit0–23 | FLASH version timestamp bits 0–23 (high bit first) |
| Byte12 | Bit0–7 | FLASH version number |

### 24.  Manufacturer Code and HW/SW Version Query  0x0017  (Bootloader version)

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 17

Data Length: 0x00

Data: None

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 17

Data Length: 0x12+N

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5–8 | Bit0–31 | Controller manufacturer code (ASCII) |
| Byte9 | Bit0–7 | Hardware version integer byte |
| Byte10 | Bit0–7 | Hardware version decimal byte (1 = 0.01) |
| Byte11 | Bit0–7 | Software version integer byte |
| Byte12 | Bit0–7 | Software version decimal byte (1 = 0.01) |
| Byte13–16 |  | Controller manufacturer code (ASCII) |
| ByteN |  | Chip ID (typically from instrument, dynamic length) |

### 25.  Instrument Backlight Brightness Query/Set  0x0018

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 18

Data Length: 0x03

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit1–7 | Reserved |
|  | Bit0 | Backlight mode: 0 = Manual dimming, 1 = Auto dimming |
| Byte6 | Bit0–7 | Manual backlight brightness percentage (0–100%) |
| Byte7 | Bit0–7 | Auto backlight brightness percentage (0–100%) |

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 18

Data Length: 0x03

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit1–7 | Reserved |
|  | Bit0 | Backlight mode: 0 = Manual dimming, 1 = Auto dimming |
| Byte6 | Bit0–7 | Manual backlight brightness percentage (0–100%) |
| Byte7 | Bit0–7 | Auto backlight brightness percentage (0–100%) |

### 26.  Night Mode Query/Set  0x0019

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 19

Data Length: 0x05

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | Night mode: 0 = Off, 1 = Auto night mode, 2 = Scheduled night mode, 3 = Manual night mode |
| Byte6 | Bit0–7 | Auto/scheduled night mode start time – hour (0–23) |
| Byte7 | Bit0–7 | Auto/scheduled night mode start time – minute (0–59) |
| Byte8 | Bit0–7 | Auto/scheduled night mode end time – hour (0–23) |
| Byte9 | Bit0–7 | Auto/scheduled night mode end time – minute (0–59) |

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 19

Data Length: 0x05

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | Night mode: 0 = Off, 1 = Auto night mode, 2 = Scheduled night mode, 3 = Manual night mode |
| Byte6 | Bit0–7 | Auto/scheduled night mode start time – hour (0–23) |
| Byte7 | Bit0–7 | Auto/scheduled night mode start time – minute (0–59) |
| Byte8 | Bit0–7 | Auto/scheduled night mode end time – hour (0–23) |
| Byte9 | Bit0–7 | Auto/scheduled night mode end time – minute (0–59) |

### 27.  Color Light Settings  0x001A

Master Side (IOT / BLE / Meter)

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | Color light mode: 0x01 = Solid color, 0x02 = Solid breathing, 0x03 = Rainbow seven-color, 0x04 = Running light effect… |
| Byte6 | Bit0–7 | (Solid color active) R 0–255 |
| Byte7 | Bit0–7 | (Solid color active) G 0–255 |
| Byte8 | Bit0–7 | (Solid color active) B 0–255 |
| Byte9 | Bit0–7 | RGB overall brightness 0–255 |

Slave Side (Controller)

| Frame Header | Device Address | Function Code | Reserved | Data Length | CRC16 |
| --- | --- | --- | --- | --- | --- |
| 0x5A | 0xXX | 0x1A | 0x00 | 0x00 | 0xXX, 0xXX |

Data: None

### 28.  Controller Software Feature Signature  0x001B

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 1B

Data Length: 0xXXX (less than 140)

Data: ASCII

### 29.  SN Serial Number Query and Set  0x001D

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 1D

Data Length: Write: 14  /  Read: 0

### 30.  Detailed Information Query  0x001E

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 1E

Data Length: 0x00

Data: None

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 1E

Data Length: Variable

Data: All in ASCII, each field separated by underscore "_"

Format:  Customer_VehicleModel_Country_VoltageCurrent Speed_HWVersion_SWVersion_Date

TC_L30_US_36V15A25KPH_H100V001_20240313

C language representation:

const u8 ControllerInfor[]={"TC_L30_US_36V15A25KPH_H100V001_20240313"};

Example:

| Field | Value | Description |
| --- | --- | --- |
| Customer Name | TC | Abbreviation of "Tao Chen" – first letter initials |
| Vehicle Model | L30 | If a suffix exists, use a dash: L30-Pro or L30Pro |
| Country (USA) | US | International standard country code abbreviation |
| Voltage/Current/Speed | 36V15A25KPH | Used for information verification |
| Hardware Version | H100 | Represents HV1.00; H0101 = HV1.01; max H99.99 |
| Software Version | V001 | Represents SV0.01; V1010 = SV10.10; max V99.99 |
| Date | (current date) |  |

### 31.  Battery Status Query  0x0020

(See TaoTao Battery Communication Protocol for details.)

### 32.  Gyroscope Angle Query  0x0021

Master Side (IOT / BLE / Meter)

Data: None

0x5A 0xXX 0x021 0x00 0x00 0xXX 0xXX

Slave Side (Controller / Instrument / Battery)

Data Length: 0x04

0x5A 0xXX 0x021 0x00 0x04 +

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit8–15 | Roll high byte (signed; 105 = 10.5°) |
| Byte6 | Bit0–7 | Roll low byte (signed; 105 = 10.5°) |
| Byte7 | Bit8–15 | Pitch high byte |
| Byte8 | Bit0–7 | Pitch low byte |

### 33.  Response Time Query/Set  0x0022

Master Side (IOT / BLE / Meter)

Data: 2 bytes

0x5A 0xXX 0x022 0x00 + 0x02 0x0X 0x0X + 0xXX 0xXX

Slave Side (Controller / Instrument / Battery)

Data Length: 0x02

0x5A 0xXX 0x022 0x00 0x02 +

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | Response type: 0 = Throttle response (range 0–10) 1 = Brake response (range 0–10) 2 = (reserved) |
| Byte6 | Bit0–7 | Value |

### 34.  BMS Firmware Version Query  0x002A

(Protocol details reserved – refer to BMS documentation.)

### 35.  Remaining Range Query  0x0030

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x0030

Data Length: 0x00

Data: None

0x5A 0xXX 0x030 0x00 0x00 0xXX 0xXX

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 30

Data Length: 0x02

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit8–15 | Remaining range high bits |
| Byte6 | Bit0–7 | Remaining range low bits |

### 36.  Current Riding Time Query  0x0031

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x0031

Data Length: 0x00

Data: None

0x5A 0xXX 0x032 0x00 0x00 0xXX 0xXX

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 31

Data Length: 0x04

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit24–31 | Riding time (unit: s) |
| Byte6 | Bit16–23 |  |
| Byte7 | Bit8–15 |  |
| Byte8 | Bit0–7 |  |

### 37.  Speed Query  0x0032

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x0032

Data Length: 0x00

Data: None

0x5A 0xXX 0x032 0x00 0x00 0xXX 0xXX

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 32

Data Length: 0x04

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit8–15 | Average speed high bits |
| Byte6 | Bit0–7 | Average speed low bits |
| Byte7 | Bit8–15 | Maximum speed high bits |
| Byte8 | Bit0–7 | Maximum speed low bits |

### 38.  Incoming Call Push  0x00A0

When there is a new incoming call, the master sends 1 (call incoming); the slave replies 0, then stops sending.

When the ongoing call is cancelled, the master sends 2 (call cancelled); the slave replies 0, then stops sending.

User info: caller number; if the caller is in the contact book, the contact name is used instead.

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 A0

Data Length: 0xXX (max 130)

Slave Side (Controller / Instrument / Battery)

| Data[0] | Data[1] | Data[2] | Data[3] | Data[4] | Data[5]~Data[n] | CRC16 |
| --- | --- | --- | --- | --- | --- | --- |
| 0x5A | 0xXX | 0xA0 | 0x00 | 0x01 |  | 0xXX, 0xXX |

Function Code: 0x00 A0

Data Length: 0x01

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | 0 = Received successfully 1 = Receive failed 2 = Call declined |

### 39.  SMS Push  0x00A1

SMS data is sent in segmented packets. The complete payload is (user info) + (\0\0) + (SMS content). If the total exceeds 130 bytes, it is split into segments; a flag bit indicates whether the current segment is the last one.

User info: SMS number; if the sender is in the contact book, the contact name is used instead.

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 A1

Data Length: 0xXX (max 130)

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit7 | Is this the last packet? 0 = Last packet, 1 = More packets follow |
|  | Bit0–6 | Packet number high 7 bits |
| Byte6 | Bit0–7 | Packet number low 8 bits |
| Byte7–… | Bit0–… | (User info) + (\0\0) + (SMS content) (Sent as a whole by time length, encoding: UTF-8) |

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 A1

Data Length: 0x03

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | 0 = Received successfully, 1 = Receive failed |
| Byte6 | Bit0–7 | Received packet number high 8 bits |
| Byte7 | Bit0–7 | Received packet number low 8 bits |

### 40.  Message Push  0x00A2

Message push app numbers:  1 = Phone,  2 = SMS,  3 = QQ,  4 = WeChat,  etc.

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 A2

Data Length: 0x02

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | Message push app number |
| Byte6 | Bit0–7 | Unread message count for this app (capped at 99) |

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 A2

Data Length: 0x01

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | 0 = Received successfully, 1 = Receive failed |

### 41.  Time Push  0x00A3  (sent once each time app connects)

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 A3

Data Length: 0x06

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | Phone time – Year |
| Byte6 | Bit0–7 | Phone time – Month |
| Byte7 | Bit0–7 | Phone time – Day |
| Byte8 | Bit0–7 | Phone time – Hour |
| Byte9 | Bit0–7 | Phone time – Minute |
| Byte10 | Bit0–7 | Phone time – Second |

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 A3

Data Length: 0x01

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | 0 = Received successfully, 1 = Receive failed |

### 42.  Bluetooth Password Query/Configure  0x00A4

Initial password: 000000

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 A4

Data Length: 0x06

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | Password digit 1 |
| Byte6 | Bit0–7 | Password digit 2 |
| Byte7 | Bit0–7 | Password digit 3 |
| Byte8 | Bit0–7 | Password digit 4 |
| Byte9 | Bit0–7 | Password digit 5 |
| Byte10 | Bit0–7 | Password digit 6 |

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 A4

Data Length: 0x01

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | Password status: 0 = Password correct / configured successfully, 1 = Password incorrect / configuration failed |

### 43.  Navigation Push  0x00A5

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 A5

Data Length: 0x07

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit7 | Navigation status: 0 = Navigation off, 1 = Navigation on |
|  | Bit0–6 | Navigation direction code |
| Byte6 | Bit0–7 | Remaining distance in current direction high 8 bits (unit: m) |
| Byte7 | Bit0–7 | Remaining distance in current direction low 8 bits (unit: m) |
| Byte8 | Bit0–7 | Total remaining distance bits 24–32 (unit: m) |
| Byte9 | Bit0–7 | Total remaining distance bits 16–23 (unit: m) |
| Byte10 | Bit0–7 | Total remaining distance bits 8–15 (unit: m) |
| Byte11 | Bit0–7 | Total remaining distance bits 0–7 (unit: m) |

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 A5

Data Length: 0x01

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | Status: 0 = Received successfully, 1 = Receive failed |

### 44.  Reset Command (Restore Factory Settings)  0x00A6

The APP only needs to send the reset command. After the slave receives it, it resets the default parameters; data is then updated to the APP via heartbeat.

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 A6

Data Length: 0x01

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | Reset command: 0 = No change, 1 = Reset parameters |

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 A6

Data Length: 0x01

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | 0 = Received successfully, 1 = Receive failed |

### 45.  Lock/Unlock Setting  0x00A7

Initial password: 123456. The password is verified first; if correct, the lock state is changed.

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 A7

Data Length: 0x07

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit7 | Lock control: 0 = Unlock, 1 = Lock |
|  | Bit0–6 | Reserved |
| Byte6 | Bit0–7 | Password digit 1 |
| Byte7 | Bit0–7 | Password digit 2 |
| Byte8 | Bit0–7 | Password digit 3 |
| Byte9 | Bit0–7 | Password digit 4 |
| Byte10 | Bit0–7 | Password digit 5 |
| Byte11 | Bit0–7 | Password digit 6 |

Slave Side (Controller)

Device Address: 0xXX

Function Code: 0x00 01

Data Length: 0x0A

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit7 | Password status: 0 = Correct / success, 1 = Incorrect / failed |
|  | Bit6 | Lock control: 0 = Unlocked, 1 = Locked |
|  | Bit0–5 | Reserved |

### 46.  Lock Password Modification  0x00A8

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 A8

Data Length: 0x0C

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | Original password digit 1 |
| Byte6 | Bit0–7 | Original password digit 2 |
| Byte7 | Bit0–7 | Original password digit 3 |
| Byte8 | Bit0–7 | Original password digit 4 |
| Byte9 | Bit0–7 | Original password digit 5 |
| Byte10 | Bit0–7 | Original password digit 6 |
| Byte11 | Bit0–7 | New password digit 1 |
| Byte12 | Bit0–7 | New password digit 2 |
| Byte13 | Bit0–7 | New password digit 3 |
| Byte14 | Bit0–7 | New password digit 4 |
| Byte15 | Bit0–7 | New password digit 5 |
| Byte16 | Bit0–7 | New password digit 6 |

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 A8

Data Length: 0x01

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit7 | Password status: 0 = Correct / success, 1 = Incorrect / failed |
|  | Bit0–6 | Reserved |

### 47.  After-Sales Password Query  0x00A9

After-sales password: used when the user forgets the password. The APP can query the device's after-sales password via this command for after-sales verification.

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 A9

Data Length: 0x00

Data: None

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 A9

Data Length: 0x06

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | After-sales password digit 1 |
| Byte6 | Bit0–7 | After-sales password digit 2 |
| Byte7 | Bit0–7 | After-sales password digit 3 |
| Byte8 | Bit0–7 | After-sales password digit 4 |
| Byte9 | Bit0–7 | After-sales password digit 5 |
| Byte10 | Bit0–7 | After-sales password digit 6 |

### 48.  Weather Information Push  0x00AA

Push weather info: includes weather code, max temperature, and min temperature (all in °C).

Weather codes:

| Code | Weather | Code | Weather | Code | Weather | Code | Weather |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Sunny (domestic day) | 10 | Showers | 20 | Sleet | 30 | Fog |
| 1 | Sunny (domestic night) | 11 | Thunderstorm | 21 | Flurries | 31 | Haze |
| 2 | Sunny (foreign night) | 12 | Thunderstorm+Hail | 22 | Light snow | 32 | Windy |
| 3 | Sunny (foreign night) | 13 | Light rain | 23 | Moderate snow | 33 | Strong wind |
| 4 | Cloudy | 14 | Moderate rain | 24 | Heavy snow | 34 | Hurricane |
| 5 | Partly cloudy (day) | 15 | Heavy rain | 25 | Snowstorm | 35 | Tropical storm |
| 6 | Partly cloudy (night) | 16 | Rainstorm | 26 | Floating dust | 36 | Tornado |
| 7 | Mostly cloudy (day) | 17 | Heavy rainstorm | 27 | Blowing sand | 37 | Cold |
| 8 | Mostly cloudy (night) | 18 | Extreme rainstorm | 28 | Sandstorm | 38 | Hot |
| 9 | Overcast | 19 | Freezing rain | 29 | Severe sandstorm | 39 | Unknown |

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 AA

Data Length: 0x28

Data (5 days × 8 bytes each = 40 bytes):

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | Day 1 – Weather code |
| Byte6 | Bit0–7 | Reserved |
| Byte7 | Bit0–7 | Reserved |
| Byte8 | Bit0–7 | Humidity |
| Byte9 | Bit5–7 | Wind direction |
|  | Bit0–4 | Wind speed |
| Byte10 | Bit4–7 | UV intensity |
|  | Bit0–3 | Air quality index |
| Byte11 | Bit0–7 | Max temperature (8-bit signed, unit °C) |
| Byte12 | Bit0–7 | Min temperature (8-bit signed, unit °C) |
| Byte13 | Bit0–7 | Day 2 – Weather code |
| … |  | (same 8-byte structure repeats for days 2–5) |
| Byte37 | Bit0–7 | Day 5 – Weather code |
| … |  | … |
| Byte44 | Bit0–7 | Day 5 – Min temperature |

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 01

Data Length: 0x0A

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | 0 = Received successfully, 1 = Receive failed |

### 49.  Query Slave (Bootloader) Upgrade Conditions  0x00D0

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 00D0

Data Length: 0x00

Data: None

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 00D0

Data Length: 0x01

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | Ready to upgrade: 0 = Ready, 1 = Not ready |

### 50.  Transmit Upgrade Data Package / Query Receive Status  0x00D1

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 00D1

Data Length: 0xXX (max 130)

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | Packet number high 8 bits |
| Byte6 | Bit0–7 | Packet number low 8 bits |
| Byte7–… | Bit0–… | Upgrade data |

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 00D1

Data Length: 0xXX (max 130)

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | Data receive status: 0 = Send next packet, 1 = Error – request re-send |
| Byte6 | Bit0–7 | Received packet number high 8 bits |
| Byte7 | Bit0–7 | Received packet number low 8 bits |

### 51.  Upgrade Verification / Upgrade End Response  0x00D2

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 00D2

Data Length: 0x04

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5–8 | Bit0–31 | CRC32 of entire upgrade file |

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 00D2

Data Length: 0x01

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | Upgrade end response: 0 = Success, 1 = Failed, 2 = In progress |

### 52.  Query Slave (Application) Upgrade Conditions  0x00E0

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 00E0

Data Length: 0x00

Data: None

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 00E0

Data Length: 0x01

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | Ready to upgrade: 0 = Ready, 1 = Not ready |

### 53.  Transmit Upgrade Data Package / Query Receive Status  0x00E1

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 00E1

Data Length: 0xXX (max 130)

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | Packet number high 8 bits |
| Byte6 | Bit0–7 | Packet number low 8 bits |
| Byte7–… | Bit0–… | Upgrade data |

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 00E1

Data Length: 0x04

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | Data receive status: 0 = Send next packet, 1 = Error – request re-send |
| Byte6 | Bit0–7 | Received packet number high 8 bits |
| Byte7 | Bit0–7 | Received packet number low 8 bits |

### 54.  Upgrade Verification / Upgrade End Response  0x00E2

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 00E2

Data Length: 0x04

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5–8 | Bit0–31 | CRC32 of entire upgrade file |

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 00E2

Data Length: 0x01

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | Upgrade end response: 0 = Success, 1 = Failed, 2 = In progress |

### 55.  Query Slave (Flash) Upgrade Conditions  0x00F0

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 00F0

Data Length: 0x00

Data: None

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 00F0

Data Length: 0x01

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | Ready to upgrade: 0 = Ready, 1 = Not ready |

### 56.  Transmit Upgrade Data Package / Query Receive Status  0x00F1

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 00F1

Data Length: 0xXX (max 130)

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | Packet number high 8 bits |
| Byte6 | Bit0–7 | Packet number low 8 bits |
| Byte7–… | Bit0–… | Upgrade data |

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 00F1

Data Length: 0x03

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | Data receive status: 0 = Send next packet, 1 = Error – request re-send |
| Byte6 | Bit0–7 | Received packet number high 8 bits |
| Byte7 | Bit0–7 | Received packet number low 8 bits |

### 57.  Upgrade Verification / Upgrade End Response  0x00F2

Master Side (IOT / BLE / Meter)

Device Address: 0xXX

Function Code: 0x00 00F2

Data Length: 0x04

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5–8 | Bit0–31 | CRC32 of entire upgrade file |

Slave Side (Controller / Instrument / Battery)

Device Address: 0xXX

Function Code: 0x00 00F2

Data Length: 0x01

| Byte | Bit | Description |
| --- | --- | --- |
| Byte5 | Bit0–7 | Upgrade end response: 0 = Success, 1 = Failed, 2 = In progress |

# 3.  Examples

| Data[0] Frame Header | Data[1] Device Address | Data[2] Function Code | Data[3] Reserved | Data[4] Data Length | Data[5]~Data[n] Data | Data[n+1] Data[n+2] CRC16 |
| --- | --- | --- | --- | --- | --- | --- |
| 0x5A | 0x21 | 0x01 | 0x00 | 0x08 |  | 0xXX, 0xXX |

Table 4 (Protocol format)

| Field Name |  | Description |  | Notes |
| --- | --- | --- | --- | --- |
| Device Address | Bit7 | Master/Slave ID | 0: Master, 1: Slave |  |
|  | Bit4–6 | Read/Write Permission | 000: Read-only 001: Write no response 010: Write with response 011: Response |  |
|  | Bit0–3 | Slave Device Address | 0001: Controller 0010: Battery 0011: Instrument |  |

Table 5 (Device Address / Read-Write Permissions)

## 3.1.  Heartbeat Data Exchange

### 3.1.1.  IOT or Bluetooth Instrument Sends

0x5A 0x21 0x01 0x00 0x08 0x00 0x00 0x00 0x00 0x00 0x00 0x00 0x00 0xXX 0xXX

0x5A : Frame header

0x21 : Device address (see Table 5)  –  0x21 = 0010 0001; bit7=0 → master; bit4-6=2 → write with response; bit0-3=1 → controller

0x00 0x01 : Function code 0x0001 = Heartbeat data exchange

0x08 : Data length = 8 bytes

0x00 × 8 : 8 bytes of 0x01 function code data content

0xXX 0xXX : CRC16 checksum, 2 bytes

### 3.1.2.  Controller Reply

0x5A 0xB1 0x01 0x00 0x0A 0x00 0x00 0x00 0x00 0x00 0x00 0x00 0x00 0x00 0x00 0xXX 0xXX

0x5A : Frame header

0xB1 : Device address (see Table 5)

0x00 0x01 : Function code 0x0001; the response function code must match the master's query function code

0x0A : Data length = 10 bytes (per 0x0001 function code table)

0x00 × 10 : 10 bytes of 0x0001 function code data content

0xXX 0xXX : CRC16 checksum, 2 bytes

## 3.2.  Query/Configure Controller Status

### 3.2.1.  IOT or Bluetooth Instrument Sends

0x5A 0x21 0x02 0x00 0x02 0x00 0x00 0xXX 0xXX

0x5A : Frame header

0x21 : Device address (see Table 5)

0x0002 : Function code – controller status

0x06 : Data length = 2 bytes (per function code table)

0x00 0x00 : 2 bytes of 0x0002 function code data content

0xXX 0xXX : CRC16 checksum, 2 bytes

### 3.2.2.  Controller Reply

0x5A 0xB1 0x02 0x00 0x02 0x00 0x00 0xXX 0xXX

0x5A : Frame header

0xB1 : Device address (see Table 5)

0x0002 : Function code – controller status

0x02 : Data length = 2 bytes

0x00 0x00 : 2 bytes of 0x0002 function code data content

0xXX 0xXX : CRC16 checksum, 2 bytes

## 3.3.  Query/Configure Controller Light Control

### 3.3.1.  IOT or Bluetooth Instrument Sends

0x5A 0x21 0x04 0x00 0x02 0x0F 0x0A 0xXX 0xXX

0x5A : Frame header

0x21 : Device address (see Table 5)

0x0004 : Function code – light control

0x02 : Data length = 2 bytes

0x0F 0x0A : 2-byte data content.  0x0F → 0000 1111,  0x0A → 0000 1010.  In the function table, 0x0F is the actual data value and 0x0A is the effective-bit declaration. Bit1 and Bit3 are effective, so the values of Bit1 and Bit3 in 0x0F are the bits to be modified; all other bits are ignored.  (Bit-control mode is specially noted in the function code table.)

0xXX 0xXX : CRC16 checksum, 2 bytes

### 3.3.2.  Controller Reply

0x5A 0xB1 0x04 0x00 0x02 0x0F 0x0A 0xXX 0xXX

0x5A : Frame header

0xB1 : Device address  –  0xB1 = 1011 0001; bit7=1 → slave; bit4-6=3 → response permission; bit0-3=1 → controller

0x0004 : Function code – light control

0x02 : Data length = 2 bytes

0x0F 0x0A : Same meaning as above – bit1 and bit3 are effective

0xXX 0xXX : CRC16 checksum, 2 bytes

## 3.4.  OTA Communication Commands

### 3.4.1.  OTA Upgrade (Optional)

OTA upgrade optionally upgrades: Bootloader program, APP program, or Flash data. The overall upgrade process is the same for all types.

Upgrade type selection:

Bootloader program: (0xD0, 0xD1, 0xD2)

Application program: (0xE0, 0xE1, 0xE2)

Flash data: (0xF0, 0xF1, 0xF2)

### 3.4.2.  Upgrade Process

1. Master sends upgrade permission query to slave.

2. Slave responds to master: upgrade permitted; enters upgrade mode.

3. Master transmits upgrade data packets to slave (packet numbers 1–65535 cyclically increasing).

4. Slave receives data packets and sends acknowledgement; if receive fails, master re-sends the failed packet.

5. After all data packets are sent, master sends upgrade verification to query completion status.

6. Slave verifies data and responds with upgrade status.

7. When master receives "upgrade complete" response, upgrade finishes and upgrade mode exits.

Note: If the master sends an upgrade permission query, data packets, or completion query and receives no response, it re-sends the current command until the slave responds correctly, or declares upgrade failed after multiple failures and exits upgrade mode.

### 3.4.3.  Mobile APP Upgrading Instrument and Controller – Supplementary Notes

1. Before upgrading, query the instrument and controller manufacturer code and hardware version to confirm the upgrade target.

2. After confirming the target, query the software version and Flash version number.

3. Currently only [1] Instrument APP, [2] Instrument Flash, and [3] Controller APP are upgraded. Other upgrade mechanisms are reserved.

4. If any version among [1], [2], [3] is detected to be lower than available firmware, an upgrade prompt is displayed.

5. Click upgrade; perform the upgrade for that firmware; re-check version info after completion.

6. If two or more among [1], [2], [3] need upgrading, follow this order: [1] → [2] → [3].

7. After each firmware upgrade completes, wait 3 seconds before starting the next firmware upgrade.

8. If the upgrade is exited or fails mid-process, exit the flow; the user may re-execute steps 1–7.

9. If the user exits during upgrade, display a "Do not exit while upgrading" prompt; the user may choose to exit, which terminates the entire upgrade flow.

10. If the user force-quits the app during upgrade, the entire upgrade flow is terminated immediately.

11. On the device side, if no upgrade command or data packet is received from the master for a long time during upgrading, the device exits back to normal operation.

### 3.4.4.  Query Whether Slave Device Is Ready to Receive Upgrade Data

Master sends  (X = D, E, or F):

| Frame Header | Device Address | Function Code | Reserved | Data Length | Data | CRC16 |
| --- | --- | --- | --- | --- | --- | --- |
| 0x5A | 0x01 | 0xX0 | 0x00 | 0x00 |  | 0xXX, 0xXX |

Slave responds  (X = D, E, or F):

| Frame Header | Device Address | Function Code | Reserved | Data Length | Data | CRC16 |
| --- | --- | --- | --- | --- | --- | --- |
| 0x5A | 0x01 | 0xX0 | 0x00 | 0x01 | 0 = Ready, 1 = Not ready | 0xXX, 0xXX |

### 3.4.5.  Send Upgrade Data

Master sends  (X = D, E, or F):

| Frame Header | Device Address | Function Code Low Byte | Function Code High Byte | Data Length | Data | CRC16 |
| --- | --- | --- | --- | --- | --- | --- |
| 0x5A | 0x21 | 0xX0 | 0x00 | N (max 130) | Byte5: Packet# high 8 bits Byte6: Packet# low 8 bits Byte7+: Upgrade data | 2 Bytes |

Slave responds  (X = D, E, or F):

| Frame Header | Device Address | Function Code Low | Function Code High | Data Length | Data | CRC16 |
| --- | --- | --- | --- | --- | --- | --- |
| 0x5A | 0xB1 | 0xX1 | 0x00 | 3 | Byte5: 0=Send next, 1=Error re-send Byte6: Received packet# high Byte7: Received packet# low | 2 Bytes |

### 3.4.6.  Upgrade Data Sending Complete – Request Upgrade Verification

Master sends  (X = D, E, or F):

| Frame Header | Device Address | Function Code | Reserved | Data Length | Data | CRC16 |
| --- | --- | --- | --- | --- | --- | --- |
| 0x5A | 0x21 | 0xX2 | 0x00 | 4 | CRC32 of entire upgrade file | 2 Bytes |

Slave responds  (X = D, E, or F):

| Frame Header | Device Address | Function Code | Reserved | Data Length | Data | CRC16 |
| --- | --- | --- | --- | --- | --- | --- |
| 0x5A | 0xB1 | 0xX2 | 0x00 | 1 | Byte5: 0=Success, 1=Failed, 2=In progress | 2 Bytes |

# 4.  Notes

## 4.1.

Before performing an OTA upgrade, query the controller manufacturer code and HW/SW version number first. The manufacturer code and hardware version number must match the upgrade file before the upgrade may proceed.

## 4.2.  CRC16 Reference Algorithm

uint16_t Crc16Calc(uint8_t *data_arr, uint8_t data_len)

{

uint16_t crc16 = 0;

uint8_t i = 0;

for(i = 0; i < data_len; i++)

{

crc16 = (uint16_t)(( crc16 >> 8) | (crc16 << 8));

crc16 ^= data_arr[i];

crc16 ^= (uint16_t)(( crc16 & 0xFF) >> 4);

crc16 ^= (uint16_t)(( crc16 << 8) << 4);

crc16 ^= (uint16_t)((( crc16 & 0xFF) << 4) << 1);

}

return crc16;

}

## 4.3.  CRC32 Reference Algorithm

const unsigned int crc32table[256] = {

0x00000000, 0x04c11db7, 0x09823b6e, 0x0d4326d9, ...

/* (full table as in original document) */

};

u32 crc32(u8 *buffer, u32 size)

{

unsigned int i, len;

unsigned int crc = 0xFFFFFFFF;

i = size % 4;

if(i != 0)

len = size + 4 - i;

else

len = size;

for (i = 0; i < len; i++)

{

if(i < size)

crc = (crc << 8) ^ crc32table[((crc >> 24) ^ buffer[i]) & 0xFF];

else

crc = (crc << 8) ^ crc32table[((crc >> 24) ^ 0xff) & 0xFF];

}

return crc;

}

0x00000000, 0x04c11db7, 0x09823b6e, 0x0d4326d9, 0x130476dc, 0x17c56b6b, 0x1a864db2, 0x1e475005, 0x2608edb8, 0x22c9f00f, 0x2f8ad6d6, 0x2b4bcb61, 0x350c9b64, 0x31cd86d3, 0x3c8ea00a, 0x384fbdbd, 0x4c11db70, 0x48d0c6c7, 0x4593e01e, 0x4152fda9, 0x5f15adac, 0x5bd4b01b, 0x569796c2, 0x52568b75, 0x6a1936c8, 0x6ed82b7f, 0x639b0da6, 0x675a1011, 0x791d4014, 0x7ddc5da3, 0x709f7b7a, 0x745e66cd, ... (full 256-entry table as listed in the original Chinese document)